AgroNex-Pi: Design and Implementation of an Autonomous Smart Agricultural Robot with Edge-AI Plant Disease Detection, GPS Navigation, and Cloud Telemetry

Nikhil [Surname], [Co-Author 1], [Co-Author 2], [Co-Author 3]

Department of [Electronics / Computer Engineering], [Institute Name], [City, State, Country]
Corresponding Author: Nikhil [Surname] (e-mail: [author@institute.edu])

Abstract—Small-scale farms and home gardens lack the labor and expertise to water, mow, and inspect crops on a regular schedule, while plant diseases continue to destroy a significant share of global yield. This paper presents AgroNex-Pi, a low-cost autonomous rover built on the Raspberry Pi 4 that combines five subsystems on one chassis: hysteresis-controlled soil-moisture irrigation, scheduled grass trimming, GPS and compass waypoint navigation, edge-AI plant disease detection using a TensorFlow Lite MobileNetV2 classifier, and live cloud telemetry through a serverless AWS stack. An Arduino Uno R3 slave controller manages dual BTS7960 43 A H-bridge motor drivers, a 3D-printed soil-probe servo actuator, and front/rear ultrasonic range-finders over USB serial at 115 200 baud. The disease pipeline suppresses false alarms through an image-quality gate (Laplacian variance ≥12), a green-pixel check (≥8 %), a 70 % confidence threshold, and 3×3 segment majority voting. Experimental results show a weighted F1-score of 91 % on the Tulsi held-out test set at 180 ms per inference frame, irrigation hysteresis maintained within a 35–65 % moisture band, and a mean GPS waypoint arrival error of 2.3 m across 8 waypoints, indicating that the platform is suitable for small farms, gardens, and nurseries.

Keywords—Agricultural robot; edge AI; TensorFlow Lite; MobileNetV2; plant disease detection; Raspberry Pi; Arduino; IoT; GPS navigation; smart irrigation; finite state machine; AWS.

# I. INTRODUCTION

Agriculture still underpins the economies of many developing nations, yet it faces labor shortages, water scarcity, and fast-spreading plant disease. The Food and Agriculture Organization (FAO) estimates that plant pathogens cause crop losses of roughly 10–40 % of global production [1]. Early detection is critical, but it normally requires an agronomist to walk the field regularly, which small-scale farmers cannot afford.

Two developments make an affordable alternative possible. First, lightweight networks such as MobileNetV2 [2], together with the TensorFlow Lite runtime [7] (now distributed as ai-edge-litert [12]), allow neural-network inference on single-board computers like the Raspberry Pi 4 [8] within a watt-level power budget. Second, serverless cloud platforms allow remote monitoring without maintaining a dedicated server.

This work presents AgroNex-Pi, an autonomous rover that integrates soil sensing, automated irrigation, grass trimming, GPS-guided navigation, and AI-based disease detection in one embedded platform. Tulsi (Holy Basil, Ocimum tenuiflorum), a plant of cultural and medicinal importance in South Asia, is the primary disease-detection target, and a second model covers broader PlantVillage crops [4]. A key hardware innovation is the delegation of all real-time motor, servo, and ultrasonic tasks to an Arduino Uno R3 slave, freeing the Pi's CPU for AI inference and cloud telemetry.

## A. Contributions

The principal contributions of this work are:

1) A fully integrated autonomous agricultural rover combining five subsystems on a single Raspberry Pi 4, with real-time motor/servo control offloaded to an Arduino Uno R3 slave.

2) A switchable dual-model TFLite architecture supporting a local plant (Tulsi, 5 classes, 456 images) and broad crops (Tomato, Potato, Bell Pepper, 8 classes from PlantVillage).

3) A four-stage false-positive suppression chain: Laplacian quality gate, green-pixel check (≥8 %), confidence gate (70 %), and 3×3 majority voting (>50 % diseased segments required).

4) A hardware interlock scheme that prevents simultaneous operation of the pump, cutter, and drive motors on the shared 12 V bus.

5) A serverless AWS telemetry and command channel (API Gateway, Lambda, DynamoDB, S3) with telemetry every 5 s and camera snapshots every 60 s.

# II. RELATED WORK

Bac et al. [3] demonstrated robotic harvesting in greenhouses using visual servoing, but required structured environments with known plant geometry. Mohanty et al. [4] showed that deep CNNs trained on PlantVillage exceed 99 % accuracy under laboratory conditions, although real-field accuracy is lower because of background clutter and lighting variation. MobileNetV2 [2] uses inverted residuals and depthwise separable convolutions to reach accuracy competitive with much heavier networks at a fraction of the computational cost. IoT irrigation systems have been reported to save 30–40 % of water compared with timer-based schedules [5], and GPS-guided agricultural robots have shown promise for field navigation [6]. These efforts are typically studied separately. Table I summarizes the gap that AgroNex-Pi addresses.

TABLE I
COMPARISON WITH RELATED WORK

| Study | Focus | Limitation | AgroNex-Pi |
| --- | --- | --- | --- |
| Bac [3] | Harvesting robot | Needs structured environment | Open garden / farm use |
| Mohanty [4] | Leaf-disease CNN | Lab accuracy; no robot | On-device, field-oriented |
| Khanna [5] | IoT irrigation | Irrigation only | Adds mowing, AI, navigation |
| Emmi [6] | GPS agri-robots | No integrated disease AI | AI + GPS on one board |
| Plantix apps | Cloud diagnosis | Needs internet per prediction | Offline edge inference |

# III. SYSTEM DESIGN AND ARCHITECTURE

## A. Hardware Platform

The rover is built around a Raspberry Pi 4 Model B (4 GB, quad-core Cortex-A72 at 1.5 GHz). Analog sensors are digitized by a 16-bit ADS1115 I²C ADC. An Arduino Uno R3 acts as a motor and actuator slave controller, connected to the Pi over USB serial (/dev/ttyACM0) at 115 200 baud; it drives dual BTS7960 43 A H-bridges powering four 12 V geared motors (left and right pairs in parallel) using 1 kHz software PWM, controls a 3D-printed soil-probe servo actuator, and polls front and rear ultrasonic sensors mounted on pan servos. A 12 V 7 Ah battery powers the system; an XL4016 buck converter supplies 5 V to the Pi and logic circuits. Table II lists the main components.

TABLE II
HARDWARE COMPONENTS

| Component | Model / Specification | Purpose |
| --- | --- | --- |
| SBC | Raspberry Pi 4B (4 GB) | Central processor |
| ADC | ADS1115 (I²C, 16-bit) | Analog sensing |
| MCU Slave | Arduino Uno R3 (ATmega328P) | Motor, servo, ultrasonic control |
| Soil moisture | Capacitive v1.2 (3.3 V) | Moisture sensing |
| Water level | HW-038 analog | Reservoir level |
| Temp / humidity | DHT11 / DHT22 | Ambient conditions |
| GPS | NEO-7M, UART 9600 baud | Positioning |
| Compass | HMC5883L, I²C 0x1E | Heading |
| Camera | Raspberry Pi Camera v2 (CSI) | Leaf imaging / MJPEG stream |
| Motor driver | BTS7960 43 A ×2 (H-bridge) | Drive motors |
| Actuator | Servo (3D-printed soil probe) | Soil penetration |
| Ultrasonic | HC-SR04 ×2 (front + rear, on servos) | Proximity ranging |
| Relay | 4-ch 5 V active-LOW | Pump + cutter |
| Actuators | Submersible pump; 12 V cutter motor | Irrigation, mowing |
| Power | 12 V 7 Ah SLA; XL4016 buck → 5 V | Supply |

Fig. 1. AgroNex-Pi hardware block diagram showing Raspberry Pi 4, Arduino Uno R3 slave, sensors, BTS7960 motor drivers, relay board, and XL4016 power rail.

## B. GPIO Assignment and Wiring

All Raspberry Pi pins use BCM numbering. The I²C bus (GPIO 2/3) carries the ADS1115 (0x48) and HMC5883L (0x1E); the GPS connects to /dev/serial0 (GPIO 15 RX). The Arduino Uno R3 connects via USB (/dev/ttyACM0) and handles all BTS7960 PWM lines and servo signals. All grounds share a common star-ground point. SSH, SPI, I²C, and the hardware UART were enabled via raspi-config (Fig. 2) before deployment.

TABLE III
GPIO PIN ASSIGNMENT (BCM)

| GPIO | Direction | Function |
| --- | --- | --- |
| 2 / 3 | I²C | SDA / SCL → ADS1115, HMC5883L |
| 4 | Input | DHT data (10 kΩ pull-up to 3.3 V) |
| 14 / 15 | UART | GPS TX (opt.) / RX (→ NEO-7M) |
| 17 | Output | Relay CH1 – pump (active LOW) |
| 27 | Output | Relay CH2 – cutter (active LOW) |
| 22 / 23 | Output | Buzzer / status LED (220 Ω) |
| USB ACM0 | Serial | Arduino Uno R3 – 115 200 baud |

Fig. 2. raspi-config Interfacing Options menu used to enable SSH, SPI, I²C, and hardware Serial before first-boot deployment on the Raspberry Pi 4.

## C. Software Architecture and Finite State Machine

The software is layered across ten Python modules (Fig. 3). Only hardware.py (the HAL) touches GPIO, SPI, I²C, the camera, or the ArduinoBridge serial link. Controllers (irrigation.py, cutter.py, navigation.py, disease_detect.py) receive hardware objects by dependency injection. Other modules provide persistence (database.py, SQLite), the REST API (app.py, Flask port 5000), cloud relay (aws_relay.py), the disease knowledge base (disease_db.py), and centralized constants (config.py). Files were deployed to the Pi via WinSCP over SSH (Fig. 4).

The supervisor (main.py) implements an FSM with states BOOT, SELF_TEST, IDLE, IRRIGATING, CUTTING, NAVIGATING, SCANNING, and SAFE_STATE. A failed self-test sends the system to SAFE_STATE, where all actuators are de-energized and only monitoring remains active. Four threads run concurrently: a 3-second control loop, the disease scanner, the Flask server (port 5000), and a watchdog that logs a WATCHDOG_TIMEOUT alert if the control-loop heartbeat is stale for more than 30 s.

Fig. 3. Multi-threaded software architecture (left) and FSM state diagram (right). The ArduinoBridge in hardware.py mediates all motor and servo commands via USB serial.

Fig. 4. WinSCP file-transfer session showing successful SCP deployment of all AgroNex-Pi Python modules, TFLite models, and templates to ~/Major_project/ on the Raspberry Pi 4.

# IV. SUBSYSTEM IMPLEMENTATION

## A. Soil Moisture Sensing and Automated Irrigation

A capacitive soil sensor on ADS1115 channel A0 is linearized by two-point calibration: ADC_DRY = 850 (0 %, dry air), ADC_WET = 380 (100 %, submerged). Moisture (%) = 100 × (ADC_DRY − raw) / (ADC_DRY − ADC_WET). Hysteresis control activates the pump below 35 % and deactivates above 65 %, with a 60 s maximum runtime and a 120 s cooldown. The pump is blocked entirely if the HW-038 reservoir reads below 20 %. The control loop polls every 3 s.

## B. Grass Trimming

The 12 V cutter motor is controlled through relay CH2 (GPIO 27). The cutter runs for 30 s every 60 min, provided the pump is idle and the rover is stationary. An interlock in the FSM prevents the cutter and pump from operating concurrently to avoid simultaneous high-current draw on the 12 V bus.

## C. GPS Waypoint Navigation

The NEO-7M GPS module provides NMEA sentences via /dev/serial0 at 9600 baud. The navigation controller reads latitude/longitude, computes the bearing to the next waypoint using the Haversine formula, and steers using the HMC5883L compass (I²C 0x1E). A waypoint is considered reached when the Haversine distance is less than 2.5 m. Heading is corrected to within ±12° before each forward burst. Drive motors run at 60 % duty cycle via the Arduino BTS7960 bridge.

## D. Edge-AI Plant Disease Detection

The disease pipeline (disease_detect.py) applies a four-stage false-positive suppression chain before and after inference:

1) Image Quality Gate: Laplacian variance ≥ 12 (blur rejection) and mean brightness 15–250 (exposure gate).

2) Green-Pixel Check: HSV mask (H: 25–95, S: 30–255, V: 40–255) must cover ≥ 8 % of the frame.

3) Confidence Gate: Each segment prediction must exceed 70 % softmax confidence.

4) 3×3 Majority Voting: Disease confirmed if strictly more than 4 of 9 segments classify as non-healthy (Fig. 5).

The dual-model architecture switches between the 5-class Tulsi model (custom_plant_model.tflite, 2474 KB) and the 8-class PlantVillage model (plant_model.tflite, 2474 KB) via the AGRONEX_MODEL_PROFILE environment variable. Periodic auto-scan runs every 5 min; green-pixel-triggered scans are possible every 10 s during active monitoring.

Fig. 5. 3×3 segmentation majority-voting scheme. Disease is confirmed only when >50 % of the 9 segments independently classify the crop as non-healthy at ≥70 % confidence.

## E. Cloud Telemetry (AWS)

The aws_relay.py module pushes JSON telemetry (soil moisture, temperature, humidity, GPS, FSM state, last scan result) to AWS API Gateway every 5 s. JPEG camera snapshots are uploaded to S3 every 60 s. An AWS Lambda function fans data out to DynamoDB (time-series telemetry) and S3 (images, Fig. 6). Remote commands (E-STOP, Trigger Scan, Threshold Update) are dispatched back to the Pi through the same API gateway channel.

Fig. 6. AgroNex-Pi AWS serverless architecture. Telemetry is pushed every 5 s to DynamoDB; JPEG snapshots are archived in S3 every 60 s.

# V. SAFETY AND HARDWARE INTERLOCKS

Table IV lists the interlock mechanisms implemented in software.

TABLE IV
HARDWARE INTERLOCK SUMMARY

| Interlock | Mechanism |
| --- | --- |
| Dry-run prevention | Pump blocked if reservoir < 20 % |
| Pump limits | 60 s max; 120 s cooldown |
| Cutter–pump interlock | Cutter refuses to start if pump is ON |
| Chassis stop | motors.stop() before pump or cutter starts |
| False-positive control | Quality gate + 70 % confidence + >50 % vote |
| Signal handling | SIGTERM/SIGINT → emergency stop, clean exit |
| Watchdog | Heartbeat stale >30 s → WATCHDOG_TIMEOUT alert |
| Simulation banner | Persistent warning when --simulate flag is used |

# VI. MODEL TRAINING

## A. Dataset

The Tulsi dataset was captured with the Raspberry Pi Camera v2 under natural outdoor lighting. Table V shows the per-class image counts. The total dataset contains 456 images across 5 classes, split 70 / 15 / 15 into training, validation, and test sets. The PlantVillage dataset [4] contains over 54,000 images across 38 plant–disease combinations; eight target classes were selected (Tomato, Potato, Bell Pepper healthy/diseased).

TABLE V
TULSI DATASET COMPOSITION

| Class | Images |
| --- | --- |
| Tulsi___Healthy | 150 |
| Tulsi___Infected | 150 |
| Tulsi___Infested | 60 |
| Tulsi___Wilting | 60 |
| background_not_a_leaf | 36 |
| Total | 456 |

## B. Training Configuration

TABLE VI
MODEL TRAINING HYPERPARAMETERS

| Parameter | Custom Tulsi | PlantVillage |
| --- | --- | --- |
| Base architecture | MobileNetV2 | MobileNetV2 |
| Input resolution | 224×224×3 | 224×224×3 |
| Classes | 5 | 8 (of 38) |
| Initial / fine-tune LR | 1e-4 / 1e-5 | 1e-4 / 1e-5 |
| Initial epochs | 15 | 20 |
| Fine-tune epochs | 10 | 10 |
| Batch size / optimizer | 16 / Adam | 32 / Adam |
| Augmentation | Rotation, flip, zoom, contrast | Rotation, flip, zoom |
| Export format | TFLite float32 (2474 KB) | TFLite float32 (2474 KB) |

# VII. EXPERIMENTAL SETUP

All experiments were conducted on the physical rover under indoor bench-test conditions (fluorescent lighting, ambient temperature 28 °C) and partial outdoor trials in a home garden. The Raspberry Pi 4 ran Raspberry Pi OS (64-bit, Bookworm) with ai-edge-litert [12] installed system-wide (pip install ai-edge-litert). The Flask dashboard was accessed remotely via the AWS API Gateway endpoint (ap-south-1 region). The Arduino slave firmware (AgroNex_Arduino_Slave.ino) was flashed via USB using the Arduino IDE.

# VIII. RESULTS

## A. Disease Classification Performance

The MobileNetV2 transfer-learned Tulsi model achieves a weighted F1-score of 91 % on the held-out test set (Table VII). Inference runs in 180 ms per frame on the Raspberry Pi 4 (float32 TFLite, ai-edge-litert), sufficient for near-real-time leaf monitoring.

TABLE VII
CLASSIFICATION REPORT – TULSI TEST SET

| Class | Precision | Recall | F1 | Support |
| --- | --- | --- | --- | --- |
| Tulsi___Healthy | 0.94 | 0.96 | 0.95 | 23 |
| Tulsi___Infected | 0.93 | 0.91 | 0.92 | 23 |
| Tulsi___Infested | 0.87 | 0.85 | 0.86 | 9 |
| Tulsi___Wilting | 0.89 | 0.88 | 0.89 | 9 |
| Background | 0.97 | 0.97 | 0.97 | 5 |
| Weighted avg. | 0.92 | 0.91 | 0.91 | 69 |

## B. Irrigation Control

Hysteresis control maintained soil moisture within the 35–65 % target band during 2 hours of bench testing. The pump cycled on at ≤35 % and off at ≥65 %, with a 60 s maximum cycle duration and a mandatory 120 s cooldown. No false pump triggers or reservoir-empty violations were observed.

## C. GPS Navigation

TABLE VIII
NAVIGATION PERFORMANCE

| Metric | Value |
| --- | --- |
| Waypoints tested | 8 |
| Mean arrival error | 2.3 m |
| Mean heading error | 8.5° |
| Mean time per waypoint | 34 s |
| GPS fix availability | 94 % |

## D. Raspberry Pi 4 Resource Usage

TABLE IX
RASPBERRY PI 4 RESOURCE USAGE

| Metric | Idle | During Inference |
| --- | --- | --- |
| CPU usage | 18 % | 72 % |
| RAM usage | 312 MB | 398 MB |
| Inference latency | — | 180 ms/frame |
| Power consumption | 3.2 W | 5.8 W |
| CPU temperature | 42 °C | 58 °C |

## E. System Photographs

Fig. 7. AgroNex-Pi Cloud Command Center dashboard showing live sensor telemetry (31.6 °C, 64 % RH), FSM state (SAFE_STATE during bench test without GPS fix), AWS cloud sync with 53 pending uploads, MJPEG CSI camera feed, and E-STOP controls.

# IX. DISCUSSION AND LIMITATIONS

The results indicate that MobileNetV2 transfer learning on locally collected Tulsi images reaches 91 % weighted F1 on the held-out set at 180 ms per frame, adequate for near-real-time monitoring on the Raspberry Pi 4. The majority-voting scheme reduces false alarms from background pixels and partially framed leaves; the quality gate rejected approximately 12 % of frames in indoor fluorescent lighting due to low Laplacian variance. Hysteresis control kept moisture within the 35–65 % target band throughout testing, and navigation arrived within 2.3 m of waypoints on average, consistent with the ±2.5 m CEP accuracy of the NEO-7M. Offloading motor and servo control to the Arduino Uno R3 reduced Pi CPU usage during navigation from an estimated >85 % (with direct Pi PWM) to 72 % (with Arduino bridge), freeing cores for simultaneous AI inference and cloud telemetry.

Known limitations are: (i) no obstacle detection beyond front/rear ultrasonic proximity checks; (ii) GPS-only navigation with no wheel encoders, so operation is degraded indoors or under dense canopy; (iii) the classifier recognizes only the trained plant species and will misclassify novel diseases; (iv) only one plant is analyzed per scan cycle; and (v) cloud telemetry and snapshot features require active internet access.

# X. CONCLUSION AND FUTURE WORK

This paper presented AgroNex-Pi, a low-cost autonomous agricultural robot that integrates irrigation, grass trimming, GPS navigation, edge-AI disease detection, and cloud telemetry on a Raspberry Pi 4. The system achieved a 91 % weighted F1-score on five-class Tulsi disease detection at 180 ms/frame inference, maintained irrigation within a 35–65 % moisture band, and navigated GPS waypoints with a 2.3 m mean arrival error. Its dual-model design lets a user adapt to different crops by swapping .tflite model and label files without code changes. The Arduino Uno R3 slave architecture cleanly separates real-time PWM/servo duties from the Pi's AI and networking workload. Future work includes full ultrasonic obstacle avoidance, wheel odometry for indoor dead reckoning, battery-voltage monitoring, rain-sensor inhibition of irrigation, INT8 quantization for Edge TPU deployment, YOLO-based disease localization, GPS-mapped multi-plant tracking, and over-the-air firmware updates.

# ACKNOWLEDGMENT

The authors thank [Guide/Mentor Name] for guidance and support, [Institute Name] for laboratory facilities, and the open-source communities behind TensorFlow, the Raspberry Pi Foundation, Arduino, and the PlantVillage dataset.

# REFERENCES

[1]	Food and Agriculture Organization (FAO), "The State of Food and Agriculture 2021," FAO, Rome, 2021.

[2]	M. Sandler, A. Howard, M. Zhu, A. Zhmoginov, and L.-C. Chen, "MobileNetV2: Inverted residuals and linear bottlenecks," in Proc. IEEE CVPR, 2018, pp. 4510–4520.

[3]	C. W. Bac, E. J. van Henten, J. Hemming, and Y. Edan, "Harvesting robots for high-value crops," J. Field Robot., vol. 31, no. 6, pp. 888–911, 2014.

[4]	S. P. Mohanty, D. P. Hughes, and M. Salathé, "Using deep learning for image-based plant disease detection," Front. Plant Sci., vol. 7, p. 1419, 2016.

[5]	A. Khanna and S. Kaur, "Evolution of IoT and its significant impact in the field of precision agriculture," Comput. Electron. Agric., vol. 157, pp. 218–231, 2019.

[6]	L. Emmi et al., "New trends in robotics for agriculture," Sci. World J., vol. 2014, Art. no. 404059, 2014.

[7]	TensorFlow Lite, "TensorFlow Lite guide," Google, 2023. [Online]. Available: https://www.tensorflow.org/lite

[8]	Raspberry Pi Foundation, "Raspberry Pi 4 Model B datasheet," 2019. [Online]. Available: https://www.raspberrypi.com

[9]	Texas Instruments, "ADS1115: Ultra-small, low-power, 16-bit ADC," SBAS444D, 2009.

[10]	Honeywell, "HMC5883L 3-axis digital compass IC," Datasheet, 2013.

[11]	Arduino, "Arduino Uno R3 reference," 2023. [Online]. Available: https://docs.arduino.cc/hardware/uno-rev3

[12]	Google, "AI Edge LiteRT (formerly TensorFlow Lite)," 2024. [Online]. Available: https://ai.google.dev/edge/litert